Experiment – 8

AIM

To study and design a 4-bit BCD adder.

PLATFORM/TOOL USED

- Simulator: https://circuitverse.org/simulator

THEORY

The digital systems handle the decimal number in the form of binary coded decimal numbers (BCD). A BCD Adder Circuit that adds two BCD digits and produces a sum digit also in BCD. BCD numbers use 10 digits, 0 to 9 which are represented in the binary form 0 0 0 0 to 1 0 0 1, i.e., each BCD digit is represented as a 4-bit binary number. When we write BCD number say 526, it can be represented as

Suppose we have two 4-bit numbers A and B. The value of A and B can vary from 0(0000 in binary) to 9(1001 in binary) because we are considering decimal numbers.

The output will vary from 0 to 18 if we are not considering the carry from the previous sum. But if we are considering the carry, then the maximum value of output will be 19 (i.e., 9+9+1 = 19).

The BCD addition procedure as follows:

- Add two BCD numbers using ordinary binary addition.

- If four-bit sum is equal to or less than 9, no correction is needed. The sum is in proper BCD form.

- If the four-bit sum is greater than 9 or if a carry is generated from the four-bit sum, the sum is invalid.

- To correct the invalid sum, add 01102 to the four-bit sum. If a carry results from this addition, add it to the next higher-order BCD digit.

Thus, to implement BCD Adder Circuit we require:

- 4-bit binary adder for initial addition

- Logic circuit to detect sum greater than 9 and

- One more 4-bit adder to add 01102 in the sum if sum is greater than 9 or carry is 1.

The logic circuit to detect sum greater than 9 can be determined by simplifying the Boolean expression of given BCD Adder Truth Table.

The Logic Diagram and Truth Table of a 4-bit BCD Adder is given in the table.

Block Diagram for BCD Adder

Truth Table for BCD ADDER

| Decimal | Binary Sum |  |  |  |  | BCD Sum |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | C’ | S3’ | S2’ | S1’ | S0’ | C | S3 | S2 | S1 | S0 |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 1 |
| 2 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 1 | 0 |
| 3 | 0 | 0 | 0 | 1 | 1 | 0 | 0 | 0 | 1 | 1 |
| 4 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 1 | 0 | 0 |
| 5 | 0 | 0 | 1 | 0 | 1 | 0 | 0 | 1 | 0 | 1 |
| 6 | 0 | 0 | 1 | 1 | 0 | 0 | 0 | 1 | 1 | 0 |
| 7 | 0 | 0 | 1 | 1 | 1 | 0 | 0 | 1 | 1 | 1 |
| 8 | 0 | 1 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 |
| 9 | 0 | 1 | 0 | 0 | 1 | 0 | 1 | 0 | 0 | 1 |
| 10 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 0 | 0 | 0 |
| 11 | 0 | 1 | 0 | 1 | 1 | 1 | 0 | 0 | 0 | 1 |
| 12 | 0 | 1 | 1 | 0 | 0 | 1 | 0 | 0 | 1 | 0 |
| 13 | 0 | 1 | 1 | 0 | 1 | 1 | 0 | 0 | 1 | 1 |
| 14 | 0 | 1 | 1 | 1 | 0 | 1 | 0 | 1 | 0 | 0 |
| 15 | 0 | 1 | 1 | 1 | 1 | 1 | 0 | 1 | 0 | 1 |
| 16 | 1 | 0 | 0 | 0 | 0 | 1 | 0 | 1 | 1 | 0 |
| 17 | 1 | 0 | 0 | 0 | 1 | 1 | 0 | 1 | 1 | 1 |
| 18 | 1 | 0 | 0 | 1 | 0 | 1 | 1 | 0 | 0 | 0 |
| 19 | 1 | 0 | 0 | 1 | 1 | 1 | 1 | 0 | 0 | 1 |

The K-map for the expression to check whether the BCD sum values greater than 9, up to decimal number 15 is given below:

K-Map for BCD Adder

| For Y |  |  |  |  |
| --- | --- | --- | --- | --- |
| I1I0 I3I2 | 00 | 01 | 11 | 10 |
| 00 | 0 | 0 | 0 | 0 |
| 01 | 0 | 0 | 0 | 0 |
| 11 | 1 | 1 | 1 | 1 |
| 10 | 0 | 0 | 1 | 1 |
| Y = S3S2+S3S1 |  |  |  |  |

For Binary sum values greater than 15, the carry bit is 1 (high), which updates the expression obtained to –

Y = C'+S3S2+S3S1

The Boolean Expressions for the conditions detecting that the Binary Sum is greater than 9 to convert it to BCD by considering the carry bit as 1 in BCD representation for the ten’s place BCD digit.

Boolean Expressions for BCD Adder

S3S1=1d  (Condition 10 and 11)

S3S2=1 (Condition for 12-15)

C'=1 (Condition for 16-19)

Logic for the checking the INVALID SUM (greater than 9)

Y= C'+S3S2+S3S1

Logic Diagram for BCD Adder

v

SIMULATED OUTPUT

RESULT

The Working of 4-bit Binary Coded Decimal (BCD) Adder combinational circuit have been analysed and the Truth Table and Boolean Expression of BCD Adder using Full Adders and OR & AND gates have been studied and verified using the online simulator.

| Criteria | Total Marks | Marks Obtained | Comments |
| --- | --- | --- | --- |
| Concept (A) | 2 |  |  |
| Implementation (B) | 2 |  |  |
| Performance (C) | 2 |  |  |
| Total | 6 |  |  |